Pre-Flight

CABIN

Master……………………………………………………...…ON

Flaps……………………………………………………..CYCLE

Fuel quantity……...…………………………..….……..CHECK

Stall Warning…………………………………………...CHECK

Pitot Heat…………………………………………………..…ON

Lights……………………………………………….…….…..ON

ARROW…………………………………………………CHECK

Current Sectional, Enroute, and plates…….………..CHECK

Flight Plan forms & Flashlights…………………….…CHECK

Fuel Tanks………………………………………………CHECK

Weight & Balance…………………………….WITHIN LIMITS

### OUTSIDE RIGHT

Flap mounts and actuating rods……………………...CHECK

Aileron movement and control check………..…...CORRECT

Wing Tip and Navigation Light……………………CORRECT

Right Fuel Tank………..……………………….BLUE & FULL

Fuel Sump…………………………………………..…...DRAIN

Main Gear Strut…………..………………………...CORRECT

Main Gear Tire……………………….INFLATION CORRECT

Tie down..……………………………...………………..CLEAR

Fuel Vent………………………………….…………….CLEAR

### ENGINE / NOSE

Propeller………………………………..……………….CLEAN

Oil……………………………………..……………….6 – 8 Qts

Cowl Inlets…………………………..………………….CLEAR

Carb Air Inlet………………………..………………….CLEAR

Fuel Sump……………………………………………….DRAIN

Tire…………………………………....INFLATION CORRECT

### OUTSIDE LEFT

Right Fuel TANK……………………………….BLUE & FULL

Fuel Sump………………………………………..……...DRAIN

Main Gear Strut…………..………………………...CORRECT

Main Gear Tire……………………….INFLATION CORRECT

Tie down..……………………………………………….CLEAR

## Fuel Vent………………………………………………..CLEAR

## Pitot Tube………………………………………CLEAR & HOT

Stall Warning Tab………………………………………CLEAR

Wing Tip and Navigation Light……………….…...CORRECT

Aileron movement and control check………..…...CORRECT

Flap mounts and actuating rods………………..…….CHECK

### TAIL & FINISH

Stabilator……..FREE & CLEAR – CORRECT MOVEMENT

Tie down..……………………………………………….CLEAR

Flaps…………………………………………………………..UP

Baggage Door………………………………………...LOCKED

Master……………………………………………………….OFF

Circuit Breakers……………………………………………….IN

Flight Brief…………………………………………COMPLETE

Normal Procedures

### START

Seats & Seat Belts…………………………..……….LOCKED

Radios and Electrical Equipment………………………...OFF

Brakes……………………………………………………….SET

Fuel selector……………………………………..Fullest Tank

Master Switch………………………………………………..ON

Rotating Beacon…………………………………………….ON

Lights…………………………………………...AS REQUIRED

Mixture………………………………………….…..FULL RICH

Carb Heat…………………………………………………COLD

Prime…………………………………………..6 COLD, 3 HOT

Aux Fuel Pump………………………………………………ON

Fuel Pressure…………………………………………..CHECK

Throttle………………………………………………..OPEN ¼”

Prop Area………………………………………………..CLEAR

Starter…………………………………………………...Engage

Magnetos………………………………………………….BOTH

Oil Pressure………………………….GREEN W/IN 30 SECS

Throttle………………………………………………...800 RPM

Aux Fuel Pump……………………………………………..OFF

### TAXI

Chocks & Tiedowns……………………………………CLEAR

Avionics Master Switch……………………………………..ON

Radios…………………SET GROUND OR CTAF & GUARD

Transponder…………………………………….…..STANDBY

Altimeter……………………………………………………..SET

Gyro………………………………………………………….SET

Clearance……………………………………….ON REQUEST

Brakes……………………………………….CHECK ON TAXI

#### TURN – TIME – TWIST – THROTTLE – TALK - TRACK

### Run UP

Fuel……………………………………………FULLEST TANK

Trim………………………………………………….TAKE OFF

Flight Controls………………………………………….CHECK

Flight instruments…………………………………………..SET

Mixture……………………………………………...FULL RICH

Throttle……………………………………………….1800 RPM

Mags…………………..CHECK Max Drop 125 / Max Diff 50

Vacuum………………………………………………….CHECK

Ammeter………………………………………………...CHECK

Oil Temperature………………………………………..CHECK

Oil pressure……………………………………………..CHECK

Carb Heat…………………………………………..RPM DROP

Throttle……………………………………………………...IDLE

Standby Vac…………………………………………….CHECK

### Take Off

Doors/Windows…………………………………………..LOCK

Transponder………………………………………………...ALT

Aux Fuel Pump………………………………………………ON

Landing lights………………………………………………..ON

Flaps…………………………………………...0-10 DEGREES

Trim Tab…………………………………SET FOR TAKEOFF

Time……………………………………………………….NOTE

### Climb

Landing lights……………………………………………….OFF

Aux Fuel Pump……………………………………………..OFF

Power…………………….…..REDUCE ABOVE 1,000 FEET

## Cruise

Mixture…………………………………...LEAN A/R with EGT

Fuel Tank………………………….SWITCH EVERY 30 MINS

Power Setting…………………75% MAX – DNE 2,500 RPM

#### TURN – TIME – TWIST – THROTTLE – TALK - TRACK

### Descent

Mixture………………………………………….AS REQUIRED

Carburetor Heat…………………………..ON AS REQUIRED

Approach Plates……………………………………….READY

### Landing

Landing lights………………………………………………...ON

Seats & Seat belts……………………………………LOCKED

Fuel……………………………………………FULLEST TANK

Aux Fuel Pump………………………………………………ON

Mixture……………………………………………………..RICH

Flaps…………………………………………….115 MPH MAX

Approach Speed……………………………………….85 MPH

Final Approach / Landing Speed…..FULL FLAPS / 76 MPH

### Roll Out

Carburetor Heat…………………………………………….OFF

Flaps…………………………………………………………..UP

Transponder………………………………...STANDBY / 1200

Radio…………………………………..SWITCH TO GROUND

### Shutdown

Landing Lights………………………………………………OFF

Avionics Master Switch……………………………………OFF

Lights………………………………………………………..OFF

Throttle……………………………………………….1500 RPM

Mixture………………………………………….IDLE CUT OFF

Magnetos……………………………………………………OFF

Throttle……………………………………………….FULL AFT

Master Switch………………………………………………OFF

Control Lock………………………………………………….ON

### SPEEDS

VR  - Rotation Speed…………………………………..60 MPH

VX  - Best Angle………………………………………..74 MPH

VY  - Best Rate…………………………………………85 MPH

VSO - Stall Dirty………………………………………….57 MPH

VS  - Stall Clean………………………………………..67 MPH

VA   - Maneuvering Speed……………………………129 MPH

VFE - Flap Extension Speed……………………….…115 MPH

## Best Glide………………………………….78 - 84 MPH

| Angle Of Bank | Flaps 40 Degrees | Flaps Retracted |
| --- | --- | --- |
| 0 o | 57 MPH | 67 MPH |
| 20 | 59 MPH | 69 MPH |
| 40 | 65 MPH | 77 MPH |
| 60 | 81 MPH | 95 MPH |

### ENGINE OUT PROCEDURES

### ENGINE FAILURE ON TAKEOFF

Master Switch.……………………………………………...OFF

Fuel Selector..….…………………………………………..OFF

Magneto Switch..…………………………………………..OFF

Flaps……………………………………………..40 DEGREES

Landing……………………………………STRAIGHT AHEAD

Touchdown……………..SLOWEST POSSIBLE AIRSPEED

### ENGINE FAILURE IN FLIGHT

Airspeed.…………………………………………….….83 MPH

Flaps.…………………………………………….……………UP

Landing Field…………………………………………..SELECT

Engine…………………………………..ATTEMPT RESTART

Fuel Selector……………………………………………CYCLE

Magneto Switch…………………………………………CYCLE

Mixture……………………………………………...FULL RICH

Aux Fuel Pump……………………………………………....ON

Carburetor Heat………………………………………...CYCLE

Transponder…………………………………..SQUAWK 7700

Radio…………….ANNOUNCE MAYDAY on 121.5 & CTAF

### Before Landing

Master Switch.……………………………………………...OFF

Fuel Selector..….…………………………………………..OFF

Magneto Switch..…………………………………………..OFF

Door……………………………………………………….OPEN

Flaps………………………………..40 DEGREES ON FINAL

Touchdown……………..SLOWEST POSSIBLE AIRSPEED

### FLY THE AIRPLANE ! ! !

### ELECTRICAL SYSTEM

### FAILURE PROCEDURES

### RADIO FAILURE PROCEDURES

Comm Radios………SWITCH BETWEEN COM 1 & COM 2

Comm Radios……………………..SWITCH FREQUENCIES

Comm Radios………………RESET SQUELCH & VOLUME

Alternator Output……………………………………….CHECK

Circuit Breakers………………………………………...CHECK

Headset Jacks & Plugs………………………………..CHECK

Speakers…… CYCLE BETWEEN HEADSET & SPEAKER

Avionics Master Switch……………………………………..ON

Master Switch………………………………………………..ON

Transponder …………………………………….SET TO 7600

Intentions For Aircraft…………………BROADCAST BLIND

Landing………………...CALL TOWER, GRND, OR CLRNC

Landing At Controlled Field…….WATCH FOR LIGHT GUN

Instrument Flight

PROCEED AS FILED OR AS DIRECTED AFTER EFCT

FLY THE AIRPLANE ! ! !

### ELECTRICAL SYSTEM

### FAILURE PROCEDURES

### ALTERNATOR FAILURE PROCEDURES

Radios………………………………….ALL OFF BUT COM 1

Lights………………………………..ALL OFF BUT BEACON

Circuit Breakers……………………………….CHECK ALL IN

Avionics Master Switch……………………………………OFF

Master Switch…………………………………………..CYCLE

Avionics Master Switch……………………………………..ON

Pitot Heat……………………………………………………OFF

Aux Fuel Pump……………………………………………..OFF

Electric Turn Coordinator……………………………...COVER

VFR………..PROCEED TO CLOSEST PRACTICAL FIELD

IFR……..DECLARE EMERGENCY – REQUEST LANDING

### ELECTRICAL SYSTEM FAILURE PROCEDURES

Avionics Master Switch……………………………………OFF

Master Switch………………………………………………OFF

Circuit Breakers……………………………….CHECK ALL IN

Electric Turn Coordinator……………………………...COVER

Master Switch…………………………………………...CYCLE

VFR………..PROCEED TO CLOSEST PRACTICAL FIELD

IFR……………DESCEND OVER BEST KNOWN TERRAIN

FLY THE AIRPLANE ! ! !

### IN FLIGHT FIRE PROCEDURES

### ENGINE FIRE PROCEDURES

Throttle………………………………………FULL FORWARD

Airspeed……………………………………………..MAXIMUM

Altitude…………………………………..DESCEND RAPIDLY

Cabin Heater………………………………………………..OFF

### IF STILL BURNING:

Mixture……………………………………………..Idle Cut-Off

Fuel Selector Valve………………………………………...OFF

Magnetos……………………………………………………OFF

Airspeed…………………………………………….MAXIMUM

### CABIN FIRE PROCEDURES

Master Switch………………………………………………OFF

Electrical Equipment………………………………………OFF

Vents……………………………………………….FULL OPEN

Altitude…………………………………..DESCEND RAPIDLY

Circuit Breakers…………………………………….CHECK IN

FLY THE AIRPLANE ! ! !

### OTHER CALAMITIES

### CABIN DOOR OPEN IN FLIGHT

Overhead Door Latch……………………………………OPEN

Lower Door Latch……………………………….PUSH DOWN

Aircraft…………………………………………SLIP TO RIGHT

Door………CLOSE BY PULLING ON TOP AND BOTTOM

### BAGGAGE DOOR OPEN IN FLIGHT

Aircraft…………………………………………SLIP TO RIGHT

Baggage………RECOVER TO BACK SEAT IF POSSIBLE

Door……………………………………………FORGET IT ! ! !

### FAMILY PET OUT OF CONTROL

Aircraft………………CLIMB IMMEDIATELY, IF POSSIBLE

Pet………………CONTAIN TO BACK SEAT IF POSSIBLE

### IF STILL NOT CONTROLLABLE

Door……………………………………………………….OPEN

## Pet………………………………………………………..EJECT

## Family………………………………………………..CONSOLE

FLY THE AIRPLANE ! ! !

### WEIGHT & BALANCE CALCULATIONS

- Add the weight of all items to the licensed empty weight.

- Use the loading graph to determine the moment of all items to be carried.

- Add the moment of all items to be loaded to the licensed empty weight moment.

- Divide the total moment by the total weight to determine C. G.

- By using the solutions of # 1 & # 4 above, locate the C.G. on the weight & range chart.

### Loading Graph

|  | ### Weight (lbs) | ### Arm Aft Datum (inches) | ### Moment (In-Lbs) |
| --- | --- | --- | --- |
| ### Licensed Empty Weight | ### 1396.3 | ### 85.84 | 119867.38 |
| ### Oil (7 quarts) | ### 13.13 | ### 32.5 | ### 426.56 |
| ### Pilot & Front PAX (340 Max) |  | ### 85.5 |  |
| ### Aft Pax (340 Max) |  | ### 118.1 |  |
| ### Fuel ( 50 Gal / 300 Lbs Max) |  | ### 95.0 |  |
| ### Baggage (200 Lbs Max) |  | ### 142.8 |  |
| ### Total Loaded Airplane |  |  |  |

IT IS THE RESPONSIBILITY OF THE PILOT AND AIRCRAFT OWNER TO ENSURE THAT THE AIRPLANE IS LOADED PROPERLY.