# Pre-Flight

Master - ON

Flaps – DOWN

Fuel quantity – CHECK

Stall Warning – CHECK

Pitot Heat - CHECK

Master - OFF

Fuel Tanks - CHECK

Oil – 6 – 8 Qts

Fuel Sumps – DRAIN 3

Struts – CORRECT

Tie downs - CLEAR

Controls - CORRECT

Baggage Door – LOCKED

Cowl Inlets - CLEAR

Flaps - UP

Seat track - LOCK

Seat belts - ON

Circuit Breakers - IN

Fight Brief - COMPLETE

# Start

Master Switch - ON

Parking Brake - ON

Rotating Beacon - ON

Throttle - ¼”

## Mixture - FULL RICH

Carb Heat - COLD

Aux fuel pump - ON

Prime – AS REQUIRED

Prop Area - CLEAR

Magnetos - START

Throttle - 800 RPM

Oil Pressure - CHECK

Avionics - ON

Lights - AS REQUIRED

Transponder - STANDBY

Radios - SET

# Run UP

Fuel - FULLEST TANK

Trim - TAKE OFF

Flight Controls - CHECK

Flight instruments - SET

Mixture – FULL RICH

Throttle - 1800 RPM

Mags – CHECK DROP

(Max 125 / Max Diff 50)

Vacuum - CHECK

Ammeter - CHECK

Oil Temperature - CHECK

Oil pressure – CHECK

Carb Heat – RPM DROP

Throttle - IDLE

Standby Vac - CHECK

# Pre-take-off

Doors/windows- LOCK

Transponder - ALT

Landing lights - ON

Flaps - 0-10 DEGREES

Aux Fuel Pump - ON

# Climb

Landing lights - OFF

Fuel Pump - OFF

# Cruise

Mixture –LEAN A/R

Fuel Tank - Every 30 Mins

# Descent

Mixture – AS REQUIRED

Carb Heat – ON A/R

# Pre-Landing

Landing lights - ON

Seat belts - ON

Fuel – FULLEST TANK

Fuel Pump – ON

Mixture - RICH

# Post-Landing

## Flaps - UP

Flaps - UP

Landing Lights - OFF

## Transponder - STANDBY

Mixture–LEAN/CUTOFF

VR Rotation	- 60 MPH

VX Best Angle	 - 74 MPH

VY Best Rate	- 85 MPH

VSO Stall Dirty	- 57 MPH

VS Stall Clean	- 67 MPH

VA                   - 129 MPH

VFE Flaps	115 MPH

Best Glide	78 MPH

# Shutdown

Avionics - OFF

Lights - OFF

Mixture - FULL LEAN

Mags - OFF

Control Lock - ON

# IN-Flight FIRE

Fuel - SHUTOFF

Mixture FULL LEAN

Master OFF

Cabin Heat/air CLOSE

110 MPH DIVE

# Engine Out

Aviate: Best Glide 74 MPH

Best Field & Assess winds

FUEL – SWITCH TANKS

Mixture - RICH

Throttle - OPEN

Fuel Pump - ON

Master - ON

Mags - CHECK

Radio - 121.5

Transponder - 7700

Aviate: Final & Shutdown

Flaps - AS REQUIRED

Mags - OFF

Master - OFF

Mixture - FULL LEAN

Fuel - SHUT OFF

Doors - OPEN

## Land